# CHAPTER 1

# FADING IN COMMUNICATION CHANNELS

In order to fully understand wireless communications, we need to have a basic idea on the characteristics of wireless channels. The behavior of a typical mobile wireless channel is considerably more complex than that of an additive white Gaussian noise (AWGN) channel. Besides the thermal noise at the receiver front end (which is modeled by AWGN), there are several other well-studied channel impairments [29] in a typical wireless channel:

Path Loss, 	which describes the loss in power as the radio signal, propagates in space

Shadowing, which is due to the presence of fixed obstacles in the propagation path of	the radio signal

Fading, 	which accounts for the combined effect of multiple propagation paths,	rapid movements of mobile units (transmitters/receivers) and reflectors.

We will give a brief introduction on these three impairments. Our focus is on fading and how spread spectrum techniques can help to combat fading in wireless channels.

1.1	Path Loss

In any real channel, signals attenuate as they propagate. For a radio wave transmitted by a point source in free space, the loss in power, known as path loss, is given by

L=4πdλ2.	(1.1)

where  λ is the wavelength of the signal, and d is the distance between the source and the receiver. The power of the signal decays as the square of the distance. In land mobile wireless communication environments, similar situations are observed. The mean power of a signal decays as the nth power of the distance:

L=cdn.	(1.2)

where c is a constant and the exponent n typically ranges from 2 to 5 [1]. The exact values of c and n depend on the particular environment. The loss in power is a factor that limits the coverage of a transmitter.

1.2 	Shadowing

Shadowing is due to the presence of large-scale obstacles in the propagation path of the radio signal. Due to the relatively large obstacles, movements of the mobile units do not affect the short-term characteristics of the shadowing effect. Instead, the natures of the terrain surrounding the base station and the mobile units as well as the antenna heights determine the shadowing behavior.

Usually, shadowing is modeled as a slowly time-varying multiplicative random process. Neglecting all other channel impairments, the received signal r(t) is given by:

r(t)=g(t)s(t),	(1.3)

where s(t) is the transmitted signal and g(t) is the random process which models the shadowing effect. For a given observation interval, we assume g(t) is a constant g, which is usually modeled [29] as a lognormal random variable whose density function is given by

p(g)=12πσgexp−lng−μ22σ2g≥00g<0.	(1.4)

>:

We notice that ln g is a Gaussian random variable with mean μ and variance σ2. This translates to the physical interpretation that μ and σ2 are the mean and variance of the loss measured in decibels (up to a scaling constant) due to shadowing. For cellular and micro cellular environments, σ, which is a function of the terrain and antenna heights, can range from 4 to 12 dB [2].

# 1.3 	Fading

Fading is the term used to describe the rapid fluctuations in the amplitude of the received radio signal over a short period of time. Fading is a common phenomenon in mobile communication channels caused by the interference between two or more versions of the transmitted signals which arrive at the receiver at slightly different times. The resultant received signal can vary widely in amplitude and phase, depending on various factors such as the intensity, relative propagation time of the waves, bandwidth of the transmitted signal etc. The performance of a system (in terms of probability of error) can be severely degraded by fading. Special techniques may be required to achieve satisfactory performance.

# 1.3.1	Parameters of Fading Channels

The following parameters are often used to characterize a fading channel:

Multipath spread Tm

Suppose that we send a very narrow pulse in a fading channel. We can measure the received power as a function of time delay as shown in Figure 1.1. The average received power P(τ) as a function of the excess time delay τ (Excess time delay = time delay - time delay of first path) is called the multipath intensity profile or the delay power spectrum. The range of values of τ over which P(τ) is essentially non-zero is called the multipath spread of the channel, and is often denoted by Tm. It essentially tells us the maximum delay between paths of significant power in the channel. For urban environments, Tm can range from 0.5μs to 5μs [29].

excess delay τ

t

Tm

transmitted impulse

average

received power

Figure 1.1 Multipath delay profile

Coherence bandwidth (Δf)c

In a fading channel, signals with different frequency contents can undergo different degrees of fading. The coherence bandwidth, denoted by (Δf)c, gives an idea of how far apart in frequency for signals to undergo different degrees of fading. Roughly speaking, if two sinusoids are separated in frequency by more than (Δf)c, then they would undergo different degrees of (often assumed to be independent) fading. It can be shown that (Δf)c is related to Tm by

Δfc≈1Tm.	(1.5)

Coherence time (Δt)c

In a time-varying channel, the channel impulse response varies with time. The coherence time, denoted by (Δt)c, gives a measure of the time duration over which the channel impulse response is essentially invariant (or highly correlated) . Therefore, if a symbol duration is smaller than (Δt)c, then the channel can be considered as time invariant during the reception of a symbol. Of course, due to the time-varying nature of the channel, different time-invariant channel models may still be needed in different symbol intervals.

Doppler spread Bd

Due the time-varying nature of the channel, a signal propagating in the channel may undergo Doppler shifts (frequency shifts). When a sinusoid of frequency is transmitted through the channel, the received power spectrum can be plotted against the Doppler shift as in Figure 1.2. The result is called the Doppler power spectrum. The Doppler spread, denoted by Bd, is the range of values that the Doppler power spectrum is essentially non-zero. It essentially gives the maximum range of Doppler shifts. It can be shown that (Δt)c and Bd are related by

Doppler shift

f

Bd

Single tone

average

received power

Δtc≈1Bd	(1.6)

Figure 1.2 Doppler power spectrum

# 1.3.2	Classification of Fading Channels

There are different types of fading according to the relation between signal and channel parameters [29]. Signal fading can be either flat or frequency selective determined comparing the signal bandwidth to the coherence bandwidth of the channel which can be defined as the maximum frequency separation for which the signals are still correlated. If the coherence bandwidth is greater than the signal bandwidth, the transmitted signal undergoes flat fading. Otherwise, the transmitted signal undergoes frequency-selective fading. On the other hand, based on Doppler spread, signal fading can be either fast or slow. Relative motion between the transmitter and the receiver results in Doppler spread in the received signal. If the Doppler spread of the channel is much less than the signal bandwidth, the signal undergoes slow fading. Practically, fast fading occurs at low data rates. Figure 1.3 shows a tree of the four different types of fading.

Flat Fading

BW of signal < BW of channel

Delay spread < Symbol period

Frequency Selective Fading

BW of signal > BW of channel

Delay spread > Symbol period

Small-Scale Fading

(Based on multipath time delay) spread)

Fast Fading

High Doppler spread

Coherence time < Symbol period

Channel variations faster than baseband signal variations

Small-Scale Fading

(Based on Doppler spread)

Slow Fading

Low Doppler spread

Coherence time > Symbol period

Channel variations slower than baseband signal variations

Figure 1.3 Types of small-scale fading

# 1.3.3	A Mathematical Model for Fading Channels

Consider a transmitted signal s(t)=Acos(2πfct) through a fading channel. Ignoring the effects of noise, the received signal can be expressed as:

r(t)=Ai=1Nαicos(2πfct+θi)	(1.7)

where the number variables αi and θi are the attenuation and the phase-shift of the ith multipath component, respectively. The above expression can be rewritten as:

r(t)=Ai=1Nαicos(θi)cos(2πfct)−i=1Nαisin(θi)sin(2πfct) 	(1.8)

We introduce two random processes X1(t) and X2(t), such that the above equation becomes:

r(t)=AX1(t)cos(2πfct)−X2(t)sin(2πfct) 	(1.9)

When there are a large number of scatterers in the channel that contribute to the signal at the receiver, X1(t) and X2(t) become Gaussian random variables with zero mean and σ2 variance according to the central limit theorem. Eq. (1.9) can be rewritten as:

r(t)=AR(t)cos(2πfct+θ(t))	(1.10)

where the amplitude R(t) and the phase θ(t) of received waveform r(t) are given by:

R(t)=X1(t)2+X2(t)2	(1.11)

θ(t)=tan−1X2(t)X1(t),	(1.12)

respectively.

Since the processes X1(t) and X2(t) are Gaussian with zero mean, the envelope of the channel response at any time instant, R(t), has a Rayleigh probability density function (pdf) and the phase is uniformly distributed in the interval [0,2].

The Rayleigh probability density function (pdf) is defined as:

PRayleigh(r)=rσ2exp−r22σ20≤r≤∞0r<0,	(1.13)

where

σ2=Er22.	(1.14)

When there are fixed scatterers or signal reflectors in the medium in addition to randomly moving scatterers, R(t) can no longer be modeled as having zero mean. In this case, the envelope R(t) has a  Rician distribution and the channel is said to be a Rician fading channel. Another distribution function that has been used to model the envelope of fading signals is the Nakagami-m distribution. These fading channel models are considered in [29,30].

The distortion in the phase can be easily overcome if differential modulation is employed. Whereas the amplitude distortion R(t) severely degrades the performance of digital communication systems over fading channels. It is usually reasonable to assume that the fading stays essentially constant for at least one signaling interval.

1.3.4	Rayleigh Fading Channel Model

Assuming the transmitted signal undergoes slow fading, the amplitude and phase of the received signal are constants during one bit duration. The received baseband signal at the base station is

rj=i=0N−1Ei/TejθiαiCij+n	(1.15)

where the amplitude Ei/Tαi has a Rayleigh distribution, the phase i is uniformly distributed between [0,2], Ai=Ei/Tejθiαi is defined as the complex parameter to be estimated, and nj is complex baseband white Gaussian noise with bandpass power spectral density N0/2=σ2 for both real and imaginary parts. Ai has a Gaussian distribution. The estimated power is the square of Ai. We assume that systems are synchronized and there is no intersymbol interference (ISI).

1.3.5	Rayleigh Fading Simulator

Clarke’s model [29] is used to generate Rayleigh fading coefficients. It is assumed that each arriving wave at the receiver has arbitrary phase and angle of arrival and equal average amplitude. There is no direct line-of-sight path and no excess delay for each wave. The Doppler shift is bounded by the maximum Doppler frequency fm:

fm=vfcc	(1.16)

where v is the vehicle speed, fc is the carrier frequency and c is the speed of light. For normal highway driving, fm is around 80 Hz. The spectral shape [29] in Clarke’s model is

IFFT

-900

| .  |2

( . )1/2

Σ

g*(N/2)-1     g (N/2)-1

g*N/2                                             g N/2

.  .

-fm               0              fm

-fm           0              fm

IFFT

g*(N/2)-1     g (N/2)-1

g*N/2                                             g N/2

.  .

-fm               0                 fm

-fm           0              fm

| .  |2

independent complex gaussian samples form line spectra

r(t)

SEZ=1.5πfm1−f−fcfm2.	(1.17)

Figure 1.4 Generation of Rayleigh fading coefficients at baseband

Figure 1.4 shows the block diagram of the Rayleigh fading simulator. The first step is to generate independent complex Gaussian random variables to form baseband line spectrum for positive frequency components. Negative frequency components are obtained by conjugating positive frequency components. Then, the line spectrum is multiplied with the spectrum SEZ. Next, inverse fast Fourier transform (IFFT) is performed on these signals obtaining the real and imaginary parts for the complex Rayleigh fading output x(t)ejθ(t), here x(t) is Rayleigh distributed and (t) is uniformly distributed in the interval or between 0 and 2 [0,2].

Figure 1.5 shows the Rayleigh fading envelopes with maximum Doppler frequencies of 10Hz and 70Hz. Slow fading is observed when the vehicle speed is low and the Doppler frequency is small. Increased vehicle speed and Doppler frequency results in faster fading.

Figure 1.5 Rayleigh fading envelopes of signals with different maximum Doppler frequencies